| United States Patent | 5,866,018 |
| --- | --- |
| Hyde , et al. | February 2, 1999 |

4. Vortex aquarium filter

Abstract

An aquarium filter is provided having a first annular chamber and an inlet or near the top of the chamber. The inlet is constructed and positioned to direct water into the chamber in a swirling direction, concentric with the annular wall. The first chamber also includes a first water outlet at the top of the chamber for filtered water to exit the chamber. The first chamber also includes a waste outlet at the bottom of the chamber. As water enters the first chamber from the inlet, it swirls in the chamber, creating a vortex. Particles within the water are forced against the annular wall, slide down the wall and migrate towards the waste outlet at the bottom of the chamber. A collector, such as a simple collection pot can be coupled to the waste outlet and particles separated from the water will collect in the pot. A selectively closeable cleaning outlet at or near the bottom of the collection pot can be used to empty the material collected there.

| Inventors: | Hyde; Robert H. (Riverview, FL), Gargas; Joseph E. (Apollo Beach, FL), Curtis; Eric W. (Tampa, FL) |
| --- | --- |
| Assignee: | The Hartz Mountain Corporation (Secaucus, NJ) |
| Appl. No.: | 08/933,117 |
| Filed: | September 18, 1997 |